How to make and bind a quilt – Advanced Method

Jacob’s Ladder Quilt

Choose 2 or 3 different fabrics.  When choosing your binding it can be the same as one of the colors you already have or a contrasting color.

|  | - Cut the following pieces - 2 pieces at 4 7/8 X 4 7/8 inches - (2 different colors these will be cut on the diagonal) - 2 strips at 2 ½ X 10 inches - 1 Strip of Binding 2 ¼ inches wide by 40 inches long |
| --- | --- |
|  | - Cut your two square on a perfect diagonal |
|  | - Put your ¼ inch foot on your machine - Set your stitch at 5, A, 2 - Check your tension on a scrap piece of fabric |
|  | - Take the 2 long strips and put them right sides together and stitch at a ¼ inch seam allowance - Make sure edges are perfectly lined up - You do not need to backstitch - Press seams to the dark side - Cut the long strip into 4 2 ½ X 2 ½ |
|  | - Match up 2 of the 2 toned squares perfectly - Sew right sides together with a ¼ inch seam allowance to create a 4x4 square. (make sure you alternate the colors) - You should now have 2 four X four squares |
|  | - Sew your diagonal seams right sides together with the 2 contrasting fabrics, ¼ inch seam allowance - DO NOT stretch the bias as you sew - Press toward the dark color |
|  | - Now you have 4 new squares in total - You need to decide how you are going to arrange your squares – this is entirely up to you |
|  | - Sew one of the diagonal squares with a 4X4 square, right sides together with a ¼ inch seam allowance - Sew the second diagonal square with the other 4X4 - Press seams to the darkest side - Sew the two new rectangles right sides together to create you final 8 ½ by 8 ½ square - BE CAREFUL to match up the middle point! |
|  | - Cut your backing and batting 1 inch larger on all fours sides as your quilt top - Use the spray adhesive and spray the backing with the wrong side facing up - Place the batting onto of the backing and smooth out - Spray the batting, place the quilt onto of the batting and smooth out - Make sure you gently pull at where the fabrics join each other to prevent puckering when you sew |
|  | - Set your pressure foot tension dial to 1 or 0 - Use a scrap fabric with batting and back to test it out. - Use a medium zigzag and stitch the outside of your quilt - Now decide what top design you are going to use and stitch the top - Stitch in the ditch, channeling or free motion - Once finished trim the backing and batting |
|  | - Fold your binding in half and press - Sew the binding to the back side of the quilt - (not like the picture below) - Line up the raw edge of your binding to the raw edge of your quilt – start just off of center |
| Going north then south for your binding | - Sew ¼ inch seam allowance - Stop just before the corner and sew on a diagonal off - Bring your binding and pull it away from you and then towards you creating an overlap - Make sure the edges of the folded over area are square to the corners - Sew again at ¼ inch seam and continue around the other 3 corners - Stop once you are ½ past the last corner |
| Extra fabric | - Overlap the two ends of the remaining binding that is not sewn by 2 ¼ inches - Sew the two binding pieces right sides together lining them up like the picture and sew corner to corner - Make sure the binding fits snug enough before you trim the excess fabric (Teacher check) |
|  | - Finish sewing the rest of the binding to the quilt - Press the binding to the other side of the quilt and pin - Top stitch from the right side of the quilt as close as you can to the edge - At the corner pull |